## 2960 Password recovery on switch

### https://aqlearningcenter.blogspot.com/2017/11/2960-switch-password-recovery-quick.html

### Step1

Power off the switch then press and hold the mode button, power on the switch and release after 15 seconds then switch should give you under mentioned prompt

switch:

Step 2

Apply command

  switch: flash_init

### Step 3

switch: load_helper

You can check the flash containing by

switch: dir flash:

then apply

switch:rename flash:config.text flash:config.old

then

switch: boot

Step 4

after reboot switch will come and prompt asking initial configurations. no for initial config

Continue with the configuration dialog? [yes/no]: N

then

Switch> enable

### Step 5

Rename the file for its original name now

#### Switch# rename flash:config.text.old flash:config.text

### Step 6

Copy the configuration into memory

Switch# copy flash:config.text system:running-config